DEVELOPMENT GUIDE

pretreatment

RF Dielectric Heating Simulation Module

Digital Replica of the QMTI GP-15 Gentle Processing Machine

Protein Separation Air Classifier Project

Simulation Framework: NVIDIA Warp (GPU-Accelerated)

February 2026

# 1. Executive Summary

This document provides the architecture, development plan, and implementation guide for the pretreatment Python module — a digital replica (digital twin) of the QMTI GP-15 Radio Frequency (RF) dielectric heating machine. The module simulates the physics of RF energy transfer, material transport via conveyor, thermal dynamics, and moisture removal from biological material (protein-rich feedstock) before it enters the milling and air classification stages of the protein separation pipeline.

Target integration: This module sits upstream in the protein separation air classifier project, providing a physics-faithful simulation of the RF pretreatment step that conditions the raw material for optimal particle size reduction and protein-starch separation.

Simulation engine: NVIDIA Warp — a GPU-accelerated, differentiable simulation framework offering JIT-compiled kernels, spatial computing primitives (HashGrid, Mesh, BVH), Finite Element Methods (warp.fem), and sparse linear algebra (warp.sparse). Warp enables real-time or faster-than-real-time simulation with full gradient support for optimization.

# 2. GP-15 Machine Anatomy for Simulation

Understanding the physical machine is essential before translating it into simulation geometry and physics. The GP-15 consists of five principal subsystems, each of which maps to a simulation domain.

| Subsystem | Physical Role | Simulation Domain |
| --- | --- | --- |
| Generator / Oscillator | Produces RF energy at 27.12 MHz via triode valve LC resonator. Anode voltage ~9.5 kV DC. | RF field source model. Not spatially simulated — modeled as a parameterized energy input function (power in kW). |
| Oven / Applicator | Rectangular chamber housing upper (movable) and lower (fixed) electrode plates. RF field applied between plates. | Primary simulation volume. 3D geometry with electrode gap as a controllable parameter. RF field distribution solved here. |
| Conveyor System | 800mm modular link belt, speed 0.1–2.0 m/min. Transports material through the oven. | Particle/material transport model. Defines residence time and material bed geometry. |
| Environment Management Unit (EMU) | Air heating (inlet filters + heating elements + blower), extraction fan removes moisture-laden air from oven. | Coupled airflow and moisture transport model. Boundary conditions for the thermal simulation. |
| Control Console (HMI/PLC) | Recipe management, electrode gap setpoint, conveyor speed, anode current monitoring, safety interlocks. | Simulation controller / parameter API. Maps directly to the module’s public interface. |

# 3. Physics Model Decomposition

The pretreatment simulation couples four physics domains. Each is modeled independently and coupled at each timestep through shared state variables.

## 3.1 RF Dielectric Heating (Volumetric Heat Source)

The GP-15 heats material via dielectric heating: the RF field at 27.12 MHz causes dipolar molecules (primarily water) to oscillate, generating internal friction and thus heat. The volumetric power density absorbed by the material is given by:

P_v = 2π · f · ε₀ · ε″_eff · |E|^2   [W/m³]

Where f = 27.12 MHz (operating frequency), ε₀ is the permittivity of free space, ε″_eff is the effective loss factor of the material (strongly dependent on moisture content), and |E| is the electric field strength between the electrodes. The field strength is controlled by the electrode gap and applied voltage.

Key modeling decisions:

- The RF field between parallel-plate electrodes is modeled as a quasi-static capacitor field (valid since wavelength >> electrode dimensions at 27 MHz).

- Fringe field effects at electrode edges are captured via an analytic correction factor or a 2D Laplace pre-solve.

- The dielectric loss factor ε″_eff is a function of moisture content and temperature, parameterized from literature or experimental data for the target protein feedstock.

- The generator is abstracted: total RF power (kW) is an input, and the field distribution is computed to match that total power dissipation.

## 3.2 Heat Transfer (Conduction + Convection)

Temperature evolution in the material bed is governed by the heat equation with the RF volumetric source term:

ρ · c_p · ∂T/∂t = ∇·(k∇T) + P_v - L_v · ṁ_evap

Where ρ is density, c_p is specific heat capacity, k is thermal conductivity (all moisture-dependent), P_v is the RF source, L_v is the latent heat of vaporization, and ṁ_evap is the local evaporation rate. The EMU airflow provides convective boundary conditions at the material surface. The GP-15 manual specifies 1 kg water removed per 1 kWh of RF energy for high surface-to-volume materials, and ~0.6 kg/kWh for low surface-to-volume feedstock — these are calibration targets.

## 3.3 Moisture Transport (Drying Kinetics)

Moisture removal is the primary process objective. The drying model tracks moisture content (wet basis) through the material bed:

∂M/∂t = ∇·(D_eff ∇M) - ṁ_evap

Where M is local moisture content, D_eff is the effective moisture diffusivity (temperature-dependent), and the evaporation rate ṁ_evap depends on local temperature, moisture content, and surface conditions. The RF heating is self-leveling: wetter regions absorb more RF energy (ε″ is higher), heat faster, dry faster, and naturally converge to uniform moisture. This is one of the GP-15’s key advantages and must be faithfully reproduced.

## 3.4 Material Transport (Conveyor Kinematics)

Material enters at the infeed, rides the conveyor belt through the oven at a controlled speed (0.1–2.0 m/min), and exits at the outfeed. The conveyor defines:

- Residence time: t_res = L_oven / v_belt (critical for total energy delivery).

- Material bed geometry: bed depth, width (800mm belt), and loading pattern.

- Steady-state assumption: for continuous production, the simulation can model a representative material slug moving through the oven, or a full Eulerian field approach.

# 4. Module Architecture

The pretreatment module is designed as a self-contained Python package that integrates into the broader protein separation project via a clean public API. Internally, it uses NVIDIA Warp for all heavy computation.

## 4.1 Package Structure

protein_separation/

pretreatment/

__init__.py                 # Public API: GP15Simulator, PretreatmentResult

config.py                   # Machine specs, material properties, dataclasses

geometry/

__init__.py

oven.py                   # Oven chamber mesh generation (electrode plates, walls)

conveyor.py               # Conveyor belt geometry and material bed mesh

attenuation_ducts.py      # Infeed/outfeed duct geometry

physics/

__init__.py

rf_field.py               # RF electric field solver (quasi-static capacitor model)

thermal.py                # Heat equation solver (FEM or FDM on Warp)

moisture.py               # Moisture transport and evaporation kinetics

airflow.py                # EMU airflow boundary conditions

kernels/

__init__.py

dielectric_heating.py     # Warp kernels: volumetric heat generation

heat_transfer.py          # Warp kernels: conduction, convection BCs

drying.py                 # Warp kernels: moisture diffusion, evaporation

transport.py              # Warp kernels: material advection on conveyor

control/

__init__.py

recipe.py                 # Recipe/preset management (mirrors HMI recipes)

controller.py             # PLC-like controller logic (electrode gap, recycling)

safety.py                 # Interlock simulation, arc detection, recycle logic

io/

__init__.py

export.py                 # Export results (VTK, CSV, NumPy)

visualization.py          # 3D and 2D visualization helpers

tests/

test_rf_field.py

test_thermal.py

test_moisture.py

test_integration.py

## 4.2 Core Data Flow

The simulation loop follows a staggered coupling scheme at each timestep:

- Material State Update — Advect material bed along conveyor direction by v_belt · dt.

- RF Field Solve — Given current electrode gap and material dielectric properties (ε″ depends on moisture, temperature), compute the electric field distribution E(x,y,z).

- Volumetric Heating — Compute P_v(x,y,z) from E-field and local ε″_eff.

- Thermal Solve — Advance temperature field by dt with P_v as source and airflow convection as boundary condition.

- Moisture Solve — Advance moisture field by dt, computing evaporation from local T and M.

- Property Update — Recompute ε″_eff, ρ, c_p, k from updated T and M for next timestep.

- Controller Step — Check anode current limits (MRH/MRL), adjust electrode gap if needed, check recycle conditions.

## 4.3 Warp Computation Strategy

The simulation domain is discretized on a structured 3D grid representing the oven interior. NVIDIA Warp provides the computational backbone:

| Computation | Warp Feature | Rationale |
| --- | --- | --- |
| RF field (Laplace/Poisson) | warp.fem on Grid3D with CG solver | Structured grid FEM with GPU-accelerated conjugate gradient. Electrode BCs as Dirichlet conditions. |
| Heat equation | warp.fem or custom FDM kernels | Explicit or implicit time integration. FEM for complex geometry; FDM kernels for structured grids (faster). |
| Moisture diffusion | Custom @wp.kernel with HashGrid | Particle-based or grid-based moisture tracking. HashGrid for neighbor queries if using SPH-like approach. |
| Material advection | Custom @wp.kernel | Simple uniform translation along conveyor axis. Boundary handling for infeed/outfeed. |
| Dielectric coupling | Custom @wp.kernel | Per-cell computation of P_v from E-field and material properties. Embarrassingly parallel. |
| Gradient/optimization | wp.Tape (autodiff) | Differentiable simulation for parameter optimization (e.g., finding optimal electrode gap + belt speed for target moisture). |

# 5. Geometry Specification

The simulation geometry is derived from the GP-15 machine dimensions specified in the manual (approximate machine envelope: 5.5 x 2.9 x 2.2 m, L x W x H). The simulation focuses on the oven/applicator region where RF processing occurs.

## 5.1 Oven Chamber (Applicator)

| Parameter | Value / Range | Notes |
| --- | --- | --- |
| Belt width | 800 mm | Modular link belt, defines material bed width. |
| Electrode gap | Variable (setpoint via HMI) | Upper electrode moves vertically. Gap controls RF power density. Encoder-tracked. |
| Electrode geometry | Parallel rectangular plates | Lower plate fixed to conveyor frame. Upper plate on motorized gearbox drive. |
| Attenuation ducts | Infeed + outfeed tunnels | Carefully dimensioned to prevent RF leakage. Defines entry/exit BCs. |
| Oven length (active RF zone) | From engineering drawings | Determines residence time at given belt speed. |

## 5.2 Simulation Domain Discretization

The 3D domain is discretized as a structured hexahedral grid (compatible with warp.fem.Hexmesh or warp.fem.Grid3D):

- X-axis: conveyor direction (infeed to outfeed), length = oven active zone.

- Y-axis: across belt width (800 mm).

- Z-axis: vertical, from lower electrode (z=0) to upper electrode (z=electrode_gap).

- Typical resolution: 50–100 cells in X, 20–40 in Y, 10–20 in Z. Tunable for accuracy vs. performance.

- Material bed occupies the lower portion of the Z-domain; air fills the gap above the material.

# 6. Key Class Designs

## 6.1 GP15Simulator (Main Entry Point)

This is the public-facing class that users of the pretreatment module interact with. It orchestrates geometry creation, physics solvers, and the control loop.

class GP15Simulator:

def __init__(self, config: MachineConfig, material: MaterialProperties, device='cuda'):

"""Initialize the digital twin with machine and material parameters."""

def load_recipe(self, recipe: Recipe) -> None:

"""Load a processing recipe (electrode gap, belt speed, RF power, extraction fan)."""

def run(self, duration_s: float, dt: float = 0.1) -> PretreatmentResult:

"""Run simulation for specified duration. Returns result with T, M fields."""

def step(self, dt: float) -> StepState:

"""Advance one timestep. For interactive / real-time use."""

def get_outlet_conditions(self) -> OutletState:

"""Get material state at outfeed. Input to downstream milling module."""

## 6.2 MachineConfig (GP-15 Specifications)

@dataclass

class MachineConfig:

rf_frequency_hz: float = 27.12e6        # Operating frequency

max_rf_power_kw: float = 15.0            # Maximum RF output power

anode_voltage_kv: float = 9.5             # HT supply voltage

belt_width_m: float = 0.8                 # 800mm modular link belt

belt_speed_range_m_per_min: tuple = (0.1, 2.0)

oven_length_m: float = 1.5                # Active RF zone length (from drawings)

electrode_gap_range_m: tuple = (0.02, 0.30)  # Min to max gap

extraction_fan_hz_range: tuple = (5, 60)  # EMU extraction speed

air_cooling_cfm: float = 1119.0           # Cooling system flow rate

## 6.3 MaterialProperties (Feedstock Characterization)

@dataclass

class MaterialProperties:

name: str = 'protein_feedstock'

initial_moisture_wb: float = 0.12          # Wet basis (e.g., 12%)

target_moisture_wb: float = 0.03           # Target 3% (from GP-15 manual example)

specific_heat: Callable[[float, float], float]  = ...  # c_p(T, M)

thermal_conductivity: Callable[[float, float], float] = ...  # k(T, M)

density: Callable[[float], float] = ...    # rho(M)

dielectric_loss: Callable[[float, float], float] = ...  # eps''(T, M)

moisture_diffusivity: Callable[[float], float] = ...    # D_eff(T)

bed_depth_m: float = 0.05                  # Material bed thickness on belt

bed_porosity: float = 0.4                  # Void fraction in bed

## 6.4 Recipe (Mirrors HMI Recipe System)

Directly maps to the GP-15 HMI Recipe Edit Screen. Up to 30 recipes can be stored, matching the machine’s limit.

@dataclass

class Recipe:

name: str

recipe_number: int                        # 1-30, 0 = manual mode

electrode_gap_mm: float                   # Setpoint from HMI

belt_speed_m_per_min: float               # Conveyor speed setpoint

rf_power_enabled: bool = True

mrh_amps: float = 2.6                     # Meter Relay High (max anode current)

mrl_amps: float = 2.0                     # Meter Relay Low (drive stop threshold)

extraction_fan_hz: float = 30.0           # Extraction fan frequency

# 7. Warp Kernel Architecture

All computationally intensive work runs as Warp GPU kernels. The kernel design follows Warp best practices: pre-allocated arrays, explicit block dimensions, and CUDA graph compatibility where possible.

## 7.1 Kernel Organization

| Kernel File | Kernels | Purpose |
| --- | --- | --- |
| dielectric_heating.py | compute_power_density, update_loss_factor | Computes P_v from E-field and ε″. Updates ε″ from T, M. |
| heat_transfer.py | heat_conduction_step, apply_convection_bc, apply_rf_source | Explicit FDM heat equation. Surface convection from EMU air. Adds P_v source. |
| drying.py | moisture_diffusion_step, compute_evaporation, update_moisture | Moisture diffusion, evaporation rate, moisture field update. |
| transport.py | advect_material, inject_infeed, collect_outfeed | Translates material fields along conveyor axis. Handles boundary injection/collection. |

## 7.2 Illustrative Kernel Snippet: Dielectric Heating

This kernel computes the volumetric power density at each grid cell. It is called every timestep after the E-field has been updated.

@wp.kernel

def compute_power_density(

e_field_mag: wp.array(dtype=float),     # |E| at each cell

eps_loss:    wp.array(dtype=float),     # eps'' at each cell

power_out:   wp.array(dtype=float),     # P_v output

freq:        float,                     # 27.12e6

eps0:        float                      # 8.854e-12

):

tid = wp.tid()

E = e_field_mag[tid]

loss = eps_loss[tid]

power_out[tid] = 2.0 * 3.14159 * freq * eps0 * loss * E * E

## 7.3 Illustrative Kernel Snippet: Explicit Heat Step

@wp.kernel

def heat_conduction_step(

T:      wp.array3d(dtype=float),        # Temperature field [nx, ny, nz]

T_new:  wp.array3d(dtype=float),        # Output temperature

P_v:    wp.array3d(dtype=float),        # RF volumetric source

rho_cp: wp.array3d(dtype=float),        # rho * c_p per cell

k:      wp.array3d(dtype=float),        # Thermal conductivity

dx: float, dy: float, dz: float, dt: float

):

i, j, k_idx = wp.tid()

# Central difference Laplacian + source term ...

# T_new[i,j,k_idx] = T[i,j,k_idx] + dt/(rho_cp[...]) * (lap + P_v[...])

# 8. Controller and Safety Simulation

The GP-15’s PLC-based control system is replicated as a discrete-event controller that runs alongside the physics simulation. This is critical for realistic behavior.

## 8.1 Electrode Gap Control

The electrode gap determines RF power delivery. The controller models:

- Homing sequence on startup (drive to upper limit, reset encoder, drive to max gap position).

- Setpoint tracking: electrode moves to recipe setpoint when RF is turned on.

- MRH protection: if anode current exceeds MRH threshold, electrode gap automatically increases (reduces power).

- MRL threshold: electrode drive stops when anode current falls below MRL.

- Movement delay: electrode position is not altered until setpoint buttons are released for 0.5s (debounce logic from manual).

## 8.2 Recycle Logic

The GP-15 has automatic recycling to recover from transient overloads (arc/flashover). The simulation models:

- Arc detection: when local E-field exceeds dielectric breakdown threshold, a ‘flashover’ event triggers.

- Recycle sequence: RF off, electrode drives up, restart delay timer (typ. 2s), then RF restores and electrode returns to setpoint.

- Recycle counter: max 4 restarts before lockout. Counter resets after a time window calculated from conveyor speed.

- Recycle lockout state: RF remains off, operator intervention required.

## 8.3 Anode Current Model

The anode current is the primary feedback signal for the operator. It is modeled as proportional to the total RF power delivered to the load:

I_a = (P_rf_total / V_anode)   # Simplified: Ia proportional to power at rated voltage

At full rated power (15 kW), I_a = 2.6 A (from GP-15 test report). The simulation computes total RF power absorbed by integrating P_v over the material volume and maps it to anode current.

# 9. Integration with Air Classifier Pipeline

The pretreatment module produces output that feeds directly into the milling and air classification stages.

## 9.1 Output Interface (OutletState)

@dataclass

class OutletState:

temperature_field: np.ndarray       # 2D cross-section at outfeed (Y, Z)

moisture_field: np.ndarray           # 2D moisture at outfeed

avg_temperature_C: float

avg_moisture_wb: float

moisture_uniformity: float           # CV or std/mean

throughput_kg_per_hr: float

total_energy_kwh: float

residence_time_s: float

The downstream milling module consumes the average moisture content and its spatial uniformity. Non-uniform drying leads to inconsistent particle sizes during milling, which degrades protein separation efficiency in the classifier. The temperature profile affects protein denaturation, which is another critical quality parameter.

## 9.2 Pipeline Integration Pattern

from protein_separation.pretreatment import GP15Simulator, MachineConfig, Recipe

from protein_separation.milling import MillSimulator

from protein_separation.classifier import AirClassifier

# Stage 1: RF Pretreatment

gp15 = GP15Simulator(config=MachineConfig(), material=my_feedstock)

gp15.load_recipe(Recipe(name='soy_protein', electrode_gap_mm=80, belt_speed_m_per_min=0.5))

result = gp15.run(duration_s=300)

outlet = gp15.get_outlet_conditions()

# Stage 2: Milling (receives pretreated material)

mill = MillSimulator(inlet_moisture=outlet.avg_moisture_wb, inlet_temp=outlet.avg_temperature_C)

# Stage 3: Air Classification

classifier = AirClassifier(particle_distribution=mill.get_output())

# 10. Development Phases

## Phase 1: Foundation (Weeks 1–3)

- Project scaffolding: Set up package structure, config dataclasses, test harness, Warp initialization.

- Geometry module: Implement oven mesh generation (Grid3D or Hexmesh), parameterized by electrode gap, belt width, and oven length.

- RF field solver: Implement quasi-static parallel-plate E-field model. Validate against analytic capacitor solution.

- Basic thermal kernel: Implement explicit FDM heat conduction with constant properties. Validate against 1D analytic solution.

## Phase 2: Core Physics (Weeks 4–6)

- Dielectric heating coupling: Connect RF field to thermal solver via P_v. Implement moisture-dependent ε″.

- Moisture transport: Implement diffusion + evaporation model. Validate self-leveling behavior.

- Property coupling: Implement temperature and moisture dependent material properties (ρ, c_p, k, D_eff, ε″).

- Conveyor transport: Implement material advection along X-axis. Infeed injection and outfeed collection.

## Phase 3: Control & Realism (Weeks 7–9)

- Controller module: Recipe system, electrode gap control with MRH/MRL logic, anode current model.

- Recycle/safety logic: Arc detection, recycle sequence with counter and lockout.

- EMU airflow model: Forced convection boundary conditions, extraction fan speed coupling, condensation avoidance logic.

- Calibration: Tune against GP-15 manual specifications (1 kg/kWh target, power/current relationships).

## Phase 4: Integration & Optimization (Weeks 10–12)

- Pipeline integration: Connect OutletState to downstream milling/classifier modules.

- Differentiable simulation: Enable wp.Tape for gradient-based optimization of recipes (minimize energy for target moisture).

- Visualization: 3D temperature/moisture field rendering. VTK export for ParaView. Dashboard for real-time monitoring.

- Performance: CUDA graph capture for steady-state loop. Profile and optimize kernel launch configurations.

## Phase 5: Validation & Documentation (Weeks 13–14)

- Validation suite: Compare simulation output against GP-15 manual examples (600 kg/hr, 4%→3% moisture, 11 kW).

- Sensitivity analysis: Sweep electrode gap, belt speed, bed depth. Document response surfaces.

- API documentation: Docstrings, usage examples, Jupyter notebooks for each physics module.

- Integration tests: Full pipeline tests with realistic feedstock parameters.

# 11. Technical Risks and Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Dielectric property data unavailable for target feedstock | Inaccurate RF heating prediction | Use literature values for similar protein isolates. Plan experimental characterization. Implement property fitting framework. |
| Numerical instability in explicit thermal solver | Simulation divergence | Enforce CFL condition: dt < dx^2 / (2α). Implement adaptive timestepping. Fall back to implicit solver (Warp CG) if needed. |
| RF field fringe effects are significant | Underestimated edge heating | Start with uniform field model, add 2D Laplace correction. Validate against known electrode geometries. |
| Warp FEM on 3D grid is too slow for interactive use | Long iteration times | Use FDM kernels for structured grid (bypasses FEM assembly overhead). Use CUDA graph capture. Reduce resolution adaptively. |
| Coupling instability between RF, thermal, and moisture | Non-physical oscillations | Use operator splitting with small enough timestep. Implement sub-cycling for fast physics. Monitor energy conservation. |
| Material bed is not a continuum (granular) | Continuum assumption breaks | For initial version, treat bed as effective continuum with porosity correction. Phase 2 can add DEM (Discrete Element Method) if needed. |

# 12. Dependencies and Environment

| Dependency | Version | Purpose |
| --- | --- | --- |
| Python | >= 3.10 | Base language |
| warp-lang | >= 1.11.0 | GPU simulation engine (kernels, FEM, sparse) |
| numpy | >= 1.24 | Array interchange, post-processing |
| scipy | >= 1.11 | Material property interpolation, validation |
| pyvista / VTK | >= 0.43 | 3D visualization, VTK export |
| matplotlib | >= 3.8 | 2D plots, dashboards |
| pytest | >= 7.0 | Testing framework |
| CUDA Toolkit | >= 11.8 | GPU acceleration (via Warp) |

Hardware recommendation: NVIDIA GPU with at least 8 GB VRAM (RTX 3070 or better). The simulation grid at moderate resolution (100x40x20 = 80,000 cells) requires minimal GPU memory, but higher resolutions and gradient computation (autodiff) scale memory requirements.

# 13. Success Criteria

- Moisture reduction from 4% to 3% at 600 kg/hr with ~11 kW RF power (matches GP-15 manual example calculation).

- Self-leveling moisture behavior: wetter regions dry faster, achieving uniform outlet moisture.

- Anode current response matches GP-15 operating curves (Ia proportional to power, 2.6A at 15 kW rated).

- Residence time = oven_length / belt_speed, with correct energy delivery scaling.

- Recycle logic correctly handles arc events with counter, lockout, and automatic restart.

- End-to-end pipeline: pretreatment outlet feeds milling module with consistent, validated material state.

- GPU simulation runs faster than real-time for the nominal grid resolution on target hardware.